R M N C / F l e x N e t

and

P C / F l e x N e t

Sysop Manual Software

June 1995

Gunter Jost, DK7WJ

translated by Mario Lorenz, DG0JAB

last edited July 1998, DL4VBP

The documentation describes RMNC/FlexNet up to version 3.3h and

PC/FlexNet up to version 3.3g.

This manual was written with great care. However, it may contain errors.

Therefore, there is no warranty of any kind nor any other legal

responsibility of the author or the translator.

Copyright (C) 1987-1998 Gunter Jost,

DK7WJ, Lichtenbergstr. 77, D-64289 Darmstadt

All rights reserved. No part of this manual may be reproduced or

duplicated without prior written consent of the author.

Versions

=========================================================================

1. News on Version 3.3

=========================================================================

1.1. Channel Synchronization

---------------------------

It is now possible to synchronize several channels by software. The DCDs

of these channels are checked mutually, and only one channel may send at

one time. The timing parameters are optimized for multimode user access

ports and cannot be changed at the moment. Activation is done by an

additional "s" in the MODE-command. All ports with the "s"-flag set are

barred against each other. A hardware DCD conjunction may, but does not

need to be removed. It is not possible to define more than one group of

synchronized channels.

1.2. DAMA-Master

----------------

The node now is capable of the DAMA protocol, currently only simplex.

On synchronized channels, the DAMA mode is synchron, too. The transmission

time per QSO is currently limited to about. 4 seconds; parameters cannot

be specified currently. DAMA mode may be activated by an "m" in the MODE-

Command. It is possible to have several independent DAMA masters at the

same time, e.g. for user ports on different frequencies. On multimode

ports, the combination "sm" has to be used (not on RMNC fullmaster).

1.3. DAMA-Slave

---------------

When a channel hears another DAMA master, e.g. when using the system as

user frontend, this channel is set to DAMA slave mode automatically. For

network nodes this feature can be turned off.

1.4. Local C-Text

-----------------

An additional C-Text can be set up for users which connect locally, i.e.

without a digipeater in their path. This text can be written by using

the command "WRITE L". Any user can read it with the help of "LOCAL".

It is useful to move special hints, for example about the local user

ports, away from the C-Text to the L-Text, if they are not of any interest

when accessing the node via interlinks.

1.5. New Link Options

---------------------

Subnet links (">" - links) can be hidden now. The ">" must be replaced by

")" to achieve this.

1.6. New Trace Options

----------------------

#       suppression of RR/RNR/REJ-Frames

$       suppression of I- and UI-Texts, i.e. only the frameheaders are

displayed

<call>  trace only this callsign. Only check source and destination

callsigns, digipeater callsigns are not checked. SSID is taken

care of, if specified.

>       only frames sent

<       only frames received

The port number still needs to be defined.

1.7. Talk Command

-------------------

With this command a line of text can be sent to another user who is

connected to the infobox. The output is similar to the "/s"-command in

convers mode. This command is not implemented on RMNC fullmasters.

Syntax: "TALK <call> <text>"

Instead of typing the TALK-command for each line, a durable TALK mode can

be activated by "TALK <CALL>"; it is ended by giving "/q". This is similar

to convers mode.

1.8. TxDelay Measurement

------------------------

On any port TxDelay measurement of the other stations can be activated.

If the measured TxDelay exceeds the needed TxDelay more than 100ms, the

connection is interrupted. For stations without a digi in their path, a

warning message is issued.

1.9. New MODE-command Parameters

--------------------------------

-     Turn off port

y     On this port, you will always be sysop as long as there are no

digipeaters in your path. For security reasons, on RMNCs this may

only be set in the MRMNC compiler. On these ports the loop detector

is inactive.

ATTENTION: This attribute should only be set on local(wired) links.

Even here, it must be impossible to connect back to the node,

otherwise everything can be reprogrammed freely.

C     Only relevant in KISS mode: Force CRC mode.

On some PC/Flexnet drivers: Activation of software DCD.

M     DAMA master (see separate discussion)

S     Channel synchronization (see separate discussion)

U     This port is a user port. Activates following features (on RMNC

fullmaster only partially implemented):

* Port never goes into DAMA-Slavemode

* TxDelay-Measurement (see separate discussion)

* If in DAMA mode: Disconnect of users who poll the digi several

times (i.e. they are no proper DAMA slaves)

If a connection is interrupted due to one of the conditions above,

a warning message is issued, but only if the user is connected

directly, i.e. without any digipeaters in the path.

1.10. New U-Options

-------------------

=       show only Infobox-QSOs (was: any character)

<call>  show only QSOs from/to this callsign (solomaster only)

*       as usual, display additional QSO data

1.11. New L-Option

------------------

With "L *", extended statistics are shown. This includes the uptime of the

software and the up- or downtime of each link. Link resets will reset this

time only if they are caused locally or the connection is interrupted.

1.12 Heardlist / MH-Command

----------------------------

The Hearlist, up to now only used internally for routing, can be displayed

now. All heard callsigns are inserted into the list. This also improves

the port routing. The list contains max. 200 callsigns and is saved to HD

on PCs every ten minutes.

Options:

<port>   Portnumber 0-15, list only callsigns heard on this port

<count>  number of callsigns to be listed. Default is 30. <count>

must be in the range 16 ... 200.

<call>   Look for this callsign. SSID is ignored if not specified.

1.13. Better command line interpreter

-------------------------------------

The command line interpreter of the infobox now works line oriented

instead of frame oriented. This makes uploading of parameters faster and

the convers mode has fewer problems formatting the text. Not implemented

on RMNC fullmasters.

1.14. Other changes to Version 3.2

----------------------------------

* Baudrate no longer defaults to 1200 baud. It now needs to be explicitly

set for each port. Therefore, the list file only shows the used ports now.

* The P-command accepts a port number; "P 1" now only lists the line

for port 1

* On solomasters, text memory is 7.5KB now

* The autorouter has been modified slightly; routing-changes in the

network are broadcasted faster now.

==========================================================================

2. INTRODUCTION

==========================================================================

2.1. The history of FlexNet

----------------------------

First ideas for the software came back in 1987, and the first version was

developed in 1988. First tests were run at home and later at the

digipeater DB0ODW in Odenwald, Germany. This digi was equipped with a

complete 6809-development system and made it possible to test new versions

by uploading them.

In 1989 the work on the RMNC version began. The RMNC (Rhein-Main-Network-

Controller) was developed by the PR-Group of Frankfurt (RMPRG) and offered

an optimal platform for FlexNet.

Since 1991 a version for MS-DOS-Computers has existed, but it has been

used only internally and never been distributed. However, after a growing

demand for such a PC/FlexNet, in 1994 a new modular driver concept was

developed in cooperation with DL8MBT, the author of the BAYCOM software.

This makes PC/Flexnet usable for almost everybody. PC/FlexNet is freely

configurable and may be used with any I/O-modules. It also offers an

application interface for high level applications, such as terminal

emulations. Development kits for this interface and the IO-drivers are

available.

PC/Flexnet is usable also for end-users which do not need a whole node.

User are offered by this an AX25-driver with the advantages of FlexNet,

like the very easy parameter setting and the modular driver concept.

It runs with almost every terminal program which supports the WA8DED

Hostmode.(See File TFEMU.DOC for details) In this case, FLEXDIGI.EXE

is not loaded, and therefore no work needs to be wasted in setting it up.

At this point we wish to thank everyone who helped us in developing the

software. Our special thanks to all the sysops, who patiently tested the

software again and again to get it stable. The goal of FlexNet was and is

to develop a robust and easy-to-maintain node software, which should annoy

sysops and users as little as possible.

2.2. Important features of FlexNet

----------------------------------

RMNC/FlexNet V2 was a version which had many differences to previous

versions for users and sysops, both operational & optical. The

RMNC/FlexNet V3.0 had some new features, where the autorouter was the most

important one. V3.2 was completely revised and became faster and more

comfortable again. V3.3 brought the DAMA channel access method and is

forerunner of a completely new access method which is also usable on echo

duplex nodes. Maintenance of the RMNC was even more simplified, the change

of the whole software of the node can now be done by changing one single

EPROM. All L2- and L1-parameters except TxDelay are set adaptively to the

current working conditions; this means that the sysop has almost no work

in setting up parameters. We found it unnecessary to have parameters for

every special cases.It was reached the point where the node runs without

any maintenance, except for dropouts, of course. Some statistics are

useful for the sysop in optimizing the interlinks and make the network

transparent to the user.

The permanent development and the very cheap hardware made RMNC/FlexNet to

the most popular system within Germany, and the growth rates in foreign

countries are impressive.

From now on FlexNet for MS-DOS is available. The RMNC is not intended to

be replaced, it is still the preferred platform for unattended nodes.

However, it is an alternative for existing PC-based nodes and makes it

possible to experiment with FlexNet without prior high investments in

hardware.

2.2.1. Hop-to-Hop-Acknowledgement

---------------------------------

Since V2 an important feature has been the immediate acknowledgement of

received frames of QSOs which run via the digi. This greatly improves the

stability of the links. The possibility of a failure used to be increased

exponentially to the number of digipeaters in the path. Now the whole link

is as good as the worst interlink on the way.

2.2.2. Connectability

---------------------

Since V2 the RMNC-digi has been connectable, i.e. QSOs are now possible

with the digipeater itself (previously it was only a L2-digi). The digi

offers several service functions, for instance convers mode, link

information and some information pages.

2.2.3 Remote Control

--------------------

The FlexNet-software makes it possible to enter sysop mode of the

digipeater remotely. This allows changing of parameters, information

pages, link information and so on after successful authorization as sysop.

2.2.4. Installation

------------------

Updating to a new RMNC/FlexNet-software version is easy, just one EPROM

needs to be programmed and changed. To simplify the configuration of the

node, an IBM-PC(and compatible) based program is available for doing this

job. Please refer to the chapters "Installing RMNC/FlexNet and "Generating

a MASTER EPROM".

The software is available at no costs for use in amateur radio and may be

copied freely in binary form (Disk/EPROM) as long as you accept the legal

terms. (see section 6)

==========================================================================

3. Using FlexNet

==========================================================================

Using a FlexNet node is very similar to using any other digipeater system.

However, you have to specify the callsign of the entry and the destination

node. The hop-to-hop-acknowledgement which has been used since V2 should

now be known to the users. The FlexNet auto-router should not cause

problems anymore also. Surprises can occur from the loop detector only. It

displays the message "loop detected" if the user tries to connect a node

which would be routed back to the same port the user comes from.

3.1. Adaptive Parameter Settings

--------------------------------

Most parameters which had to be set manually on traditional node systems

are set automatically by FlexNet according to the actual conditions. After

many experiments, RETRIES were set to 10, but this caused QSOs on fast

links to break on short interrupts. To fix this, the node polls at least

90 seconds before giving up. The number of polls, therefore, indirectly

depends on the data rate and the use of the channel.

FRACK (Frame-Acknowledge-Time, T1) is permanently adjusted to the time the

other station needs to acknowledge the I-Frames.

MAXFRAME is set according to the reliability of the connection. When all

frames are acknowledged reliably, MAXFRAME is quickly set to 7.

On 1200Bd, however, 7 frames are sent relatively rarely, since the

maximum time of one transmission is restricted to 12 seconds. All QSOs are

handled equally, i.e. they share the 12 seconds transmission time. If one

station needs a retransmission of frames (e.g. by REJects), MAXFRAME is

decreased instantly. In this case it is provable that the throughput is

significantly higher when MAXFRAME is set between 1 and 3. Only the

quality of the connection TO the other station is checked, not the

connection FROM the other station. A poll which gets the acknowledgement

of all frames or a lost acknowledgement does not affect MAXFRAME.

PERSISTENCE is the most critical parameter when many users are using the

same channel. A fixed parameter setting is always only a bad compromise

between collision probability and transmission willingness.

FlexNet, therefore, makes use of a new access method, which works fully

adaptively. All stations on the channel are permanently counted (column

"usr" in the P-list). A waiting time is calculated from this value and

from the data rate. The channel needs to be free for at least this time

(about 4 secs on 1200Bd and 10 users) to cause FlexNet to think about

transmitting. When this time is over, the channel is checked in variable

intervals and - if necessary - the node goes "on air". The throughput is

drastically increased by this algorithm, especially on duplex nodes. Now

aggressive stations do not have the advantage of being serviced faster;

there is always enough free time for weak stations. If there are only one

or two stations, the channel is almost fully accessed, thus making fast

downloads possible on low traffic times. On simplex nodes, of course, many

collisions still happen.

But even here, the new algorithm improves things significantly.

It would be good if the user software could follow this trend to adapt the

parameters to the actual conditions. A computer should be able to do this

job better and faster than a human could do, especially since it seems to

be common that most users are unable to set their parameters correctly.

3.2. The phases of a connection via the Digipeater

--------------------------------------------------

3.2.1. Link Setup

-----------------

During the connection setup the received SABM-frames are routed normally,

the UA is not sent immediately. This means that the connection is

established only if the called station is reachable. When the called

station answers with an UA, this will be routed to the calling station. At

this moment, 2 QSOs are in the user list, one from the calling station to

the called station, and one back. A special feature of FlexNet is, that

the connection can be established in one single step without

disadvantages, the common L2 digipeating is simulated. At home, you can

directly "Connect <user> via <entrynode> ,<exit node>" or "Connect

<destination node> via <entry node>". You do not need several "Connect"

commands to make your way to the destination. As hop-to-hop-

acknowledgement is still active, the quality of your QSO is not affected.

3.2.2. Information Transfer

---------------------------

During the information transfer, hop-to-hop-acknowledgement gets

activated. Every I-Frame is immediately acknowledged by the digipeater.

Now the digipeater tries to route the packet further on to its

destination. REJects due to interference or collision only happen at one

section, not in the whole route.

3.2.3. Link Failure

-------------------

If the connection breaks at one side during information transfer, the

digipeater interrupts the whole connection and displays the message:

"*** OK0NE: link failure". Thus, the partner gets informed that something

does not work.

3.2.4 Disconnect

----------------

When station "A" disconnects, the digipeater responds immediately with an

UA. The digi now tries to abort the connection to "B". When there is not

any data for station "B" anymore, this happens at once. If there are some

I-frames left, they are sent to "B" before the connection is terminated.

When "B" tries to send frames to "A", which is already disconnected, these

frames are discarded.

3.3. Methods of Routing

-----------------------

As mentioned above, not every digipeater in between needs to be named,

only the entry- and the destination-digipeater must be known. There are

4 methods to route a frame to its destination. They are tried in the

following order:

1. destination table routing

2. link table routing

3. Heardlist routing

4. SSID routing

3.3.1. Destination Table Routing

--------------------------------

The first method is based upon the callsign information. The digi tries

to look up the destination callsign in a table maintained by the

autorouter. If the callsign is found in this list, the frame is sent to

the correspondingneighbor.

Example: DB0ODW has the following interlinks:

1:DB0KT

2:DB0AAC

3:DB0IE

and knows the destinations: DB0EQ, DB0ZDF etc.

The frame: <fm DG3FBL to DK7WJ via DB0ODW, DB0ZDF>

gets expanded to <fm DG3FBL to DK7WJ via DB0ODW*,DB0AAC,DB0ZDF>

DB0AAC is now the next call in the digipeater field, and is known as a

neighbor of DB0ODW on port 2. So the frame is sent on port 2 to DB0AAC

which will then route it to DB0ZDF.

3.3.2. Link Table Routing

-------------------------

When the router does not find a matching entry in the destination table,

the digi now tries to look up the call in the link table as set up by the

sysop. If a route is found here, the frame is sent to the right port.

Example: DB0ODW has the following interlinks:

1:DB0KT

2:DB0AAC

3:DB0IE

4:DB0AIS #

The frame <fm DG3FBL to DK7WJ via DB0ODW,DB0AIS>

is transmitted on port 4, although there is no entry in the destination

table. In this case, only the link table is used for routing.

3.3.3. Heardlist routing

------------------------

The node remembers the last 200 stations heard in an internal list. The

SSID of the station is taken care of, if possible. So it is possible to be

standby on different ports with different SSID's without any problems; you

will always receive your frames on the right port. Incoming connection

wishes and UNPROTOS are routed according to this list.

3.3.4. SSID routing

-------------------

The last chance to get the frame delivered is the SSID specified for the

digipeater. The sysop can assign SSID's to certain ports using the PARMS

command. To make use of the SSID routing, the user just specifies the SSID

of the port where he wishes his frame to be transmitted.

Example:

DB0ODW has the following mappings of the SSID's to the port numbers:

port 1 has the SSID -0, port 5 the SSID -12, port 6 the SSID -3

When there is a connect request received which specifies a SSID for the

node, the frame is sent on the port specified by the SSID:

<fm DG3FBL to DK7WJ via DB0ODW-12>

This frame is transmitted on port 5, which owns the SSID -12, provided

there is no other way known to DK7WJ. On port 5 the following frame

appears:

<fm DG3FBL to DK7WJ via DB0ODW-3*>

This ensures that someone on port 5 knows where the frame came from, i.e.

the entry-SSID is put into the frame. This SSID change is needed to ensure

that the receiver knows where to send his UA to, i.e. port 6 with the SSID

-3. This is an important feature of FlexNet. All paths are reversible,

i.e. transparent to the receiver, even if the connection came by

connecting further on at a node.

This routing method is used mainly on user ports.

When all these routing methods fail, the frame is discarded. If the frame

was created by an "Connect" command at the node, the user gets the

message:   "*** <call>: can't route"

==========================================================================

4.Infobox-commands

==========================================================================

4.1. User Commands

------------------

User commands are all the commands normal users can access. The sysop has

a set of additional commands or may specify additional parameters to

normal user commands. In this documentation, <CR> means entering of a

Carriage Return, $0D. The "=>" is the system prompt of FlexNet;input is

expected now. All input can be made either upper or lower case.Is another

command entered than those listed below, the node answers with:

"invalid command".

4.1.1. L<A>test News

--------------------

Syntax: A <CR>

The A-Command shows the text for latest news as set by the sysop. After a

cold reboot this text is empty.

4.1.2. <B>eacon

---------------

Syntax: B <CR>

The B-Command shows the current beacon file. In this file you can see

which beacon is sent on which port in which interval. After a cold reboot

the default beacon is sent on port 0 or 1.

4.1.3. <C>onvers mode

---------------------

Syntax: C <CR>

If no callsign is given, the CONNECT command puts you in convers mode. By

this mode, a great number of stations can have a round table conversation

There are 255 different convers channels available. After entering the

C-Command, you get a list of all stations connected to the node and, if

they are in convers mode, too, the channel on which they are. Now the node

prompts for a number, which selects the channel you want to join.

Example:

=>C <CR>

users:

0: DL1AA 0:DL1ZZ  ---: DL2XY    73: DG3FBL    73: DK7WJ

channel ? 73 <CR>

*** starting convers, exit: /q

In this example, DL1AA and DL1ZZ are on channel no. 0 and DG3FBL and DK7WJ

on channel 73. DL2XY is connected to the node without being in convers

mode. Having given the desired number 73, the conversation starts. All

stations logged in onto the chosen channel get the message:

" <DL9ABC>: *** Logon"

While being in convers mode you have the following commands at your

disposal:

"/w"              shows all stations connected to the node (with convers

channel number if available)

"/c"              shows the actual channel number

"/c n"            switches to channel n

"/s <call> <msg>" sends private msg to <call> only

"/m <call> <msg>" sends private msg to <call> only

"/q"              quits convers mode

If a station disconnects while being in convers mode or quits convers

mode, all other users of the channel get the message:

"<DL9ABC>: *** Logoff".

If a user changes to another channel, the users of the left channel get

the message:

"<DL9ABC>: *** switched to channel n"

If there is no channel number entered on convers start-up, convers mode is

ended immediately. You are then prompted for a new command.

4.1.4. <C>onnect

----------------

Syntax: C Call [via] [digi1 digi2 ... digi8] <CR>

The CONNECT command is used to connect further onwards. The node will try

to connect you to the station via the path you specified. To confirm your

command, you get the message "link setup...".As soon as the connection is

made, you will get "*** connected to <call>" from the node. When the

called station did not respond, you get "*** failure with <call>". If the

called station sends a Busy (DM),  the message "*** busy from <call>" is

sent to you.

The link setup can be interrupted by sending a single <CR> to the node.

If you see the message "*** can`t connect twice", you have tried to

establish a QSO which already exists with the same callsign fields.

With the C-Command it is also possible to change the user port, if the

node has more than one. By typing  "C -7" you change to the port with the

SSID 7. This is acknowledged by the message "*** <call>: SSID OK".

If you connect to another station from the node onwards, and that station

disconnects you, you will get reconnected to the node. To show you what

happened, you get a "*** reconnected to <call>" then.

A connect request will be denied, if it causes a loop in the network.

If, for example, you are connected to DB0KT via DB0ODW, you cannot connect

back to DB0ODW nor to other nodes behind DB0ODW.  You should quit the QSO

with DB0KT then and retry after the reconnect.

Example: (user is connected to DB0HP)

=> C DB0ODW <CR>

link setup...

*** connected to DB0ODW

RMNC/FlexNet V3.3d - DB0ODW - JN49 IQ - Help mit H

=> C DB0HP <CR>

*** DB0ODW: loop detected

=> Q <CR>

73!

*** reconnected to DB0HP

=>

4.1.5. <D>estinations

---------------------

Syntax: D [call] <CR>

The DESTINATIONS command prints out the destination table maintained by

the node. In this table all nodes, where the autorouter knows a way to,

are shown. For every callsign there are the SSID range of the callsign and

the average round trip time in 100 ms steps are shown. As an optional

parameter a destination callsign may be given. The node will now try to

work out the way to this node and will show it (after some seconds,

depending on the (round-trip time). Uppercase callsigns mean that the node

knows the FlexNet protocol, lower case callsigns are inserted by the

autorouter to reach the next FlexNet node. The characters "???" mean, that

the previous digi does not know the way to the destination. This may

happen, when the route to the destination is reorganized at the moment or

when the destination is not reachable anymore. The "D-Table" is usually

the same on all nodes. Only when round trip times get too high, a node is

not shown anymore. Only nodes that you can reach without link loops are

shown by default. This reduces link load and has the advantage that you

will see only the nodes that are not in your direction. By using the

option "*", you will get the complete list. Another possibility is the

selective display of a part of the list. By entering "D HB9" for example,

you get all destinations containing "HB9" in the callsign, i.e. the whole

Swiss network. Both parameters may be used together. If you type "D * HB9"

you will get all Swiss destinations, including these you cannot reach without

loops.

4.1.6. <F>ind

-------------

Syntax: F call <CR>

With the FIND command it is possible to look for a station which is

standby on this or another node. When the F-Command and the callsign are

entered, the digi sends UI-frames with the POLL-bit set to this station

via some neighbor nodes. Source callsign is the callsign of the OM who

issued the FIND command. If the called station hears the frame, it will

answer with a DM- Frame. The node analyses all frames coming back and is

able to determine if this was an answer of the FIND command. If this is

the case, you will get a message via which node the station was found. If

the called station is already connected to the node, no special frame is

sent and the user will get the message that the user is QRV on the digi.

Example:

=>F DK7WJ <CR>

*** DK7WJ found via DB0ODW

=>

Only the node via which the called station was found is put out. It will

be known to the autorouter. If the station was not found, a system prompt

"=>" appears again. Since the used UI and DM frames may get lost, it is

advisable to use the FIND command more than only once to be sure the user

is not QRV. Due to the protocol, the SSID of the called station must be

known.

4.1.7. <H>elp

-------------

Syntax: H <CR>

The H-Command prints out the text-file HELP. The text can be entered by

the sysop only and should give short help text about using the node. After

a cold reboot the text is empty.

4.1.8. <I>nfo

-------------

Syntax: I <CR>

The I-Command prints out the text-file INFO. This text can be entered by

the sysop only and should provide information about the node (QTH,

equipment, antennas and so on). After a cold reboot the text is empty.

4.1.9. <IO> (In/Out)

--------------------

Syntax: IO <CR>

The IO-Command shows the state of the I/O-ports of the RMNC reset card.

There are 16 lines in and 16 lines out. The latter may be set only by the

sysop. Using this ability it is possible to remote control the node by

hardware. There are no limits to the fantasy of the sysop. The data is

shown binary.

Example:

=>IO<CR>

I: 0000 0000 0000 0000     O: 0000 0000 0000 0000

=>

The input lines are shown first and then those of the output. 0 means

"low", 1 means "high". The meaning of the single bits needs to be

documented by the sysop.

4.1.10. <L>inks

---------------

Syntax: L <CR>

The LINKS-Command displays the link table set up by the sysop.

Example:

=>L <CR>

DB0KT     0-7     60/68        P1

DB0AAC    0-15    (---)        P2

DB0IE     0-1      583         P3 @

DB0EQ     0-8   (355/399)      via DB0IE

DK7WJ     8-11    44/67        P0  -

DB0ABA                         P4

DB0BBS    0-15     ---         P5

In the first column the callsigns of the neighbor nodes are shown. Second

column shows the SSID ranges of these stations (default: 0-15). In the

third column you read the round trip time to the neighbor in 100 ms -

steps. No number here means that the round trip time is not calculated.

Three hyphens mean that the link is not available at the moment. Three

hyphens within brackets mean that the link is not available but the

autorouter is aware of another way to the station. If there is only one

number in the column, the link partner does not know about the FlexNet

protocol, or the internode QSO could not be established. When the sysop

knows that the neighbor does not know the FlexNet protocol, he may set the

attribute "@" to the link. Then only the link is tested, not if the

partner knows the protocol. If the round trip time is surrounded by

brackets, the link is so bad that it is not made known to the network. If

there are two numbers, separated by a diagonal stroke, the neighbor is a

FlexNet node. In this case the round trip times of both directions are

shown. If these values are within brackets, the autorouter knows a better

way to the destination, i.e. the direct link is not used. The 4th column

shows either the port number of the link to the neighbor (on direct links)

or the stations via which the neighbor is reachable. A hyphen behind the

port number means that the link is not made known to the network. This may

be used for temporary links or software tests for example.

"L *" shows the last 16 round trip times that have been calculated. This

makes the diagnosis of problems more easy.

4.1.11. <LO>cal

---------------

Syntax: LO <CR>

The LO-Command shows the text-file LOCAL. This text is appended to the

CTEXT for local users, but it can be displayed by the LO command

separately. The text may only be entered by the sysop. After a cold reboot

this text is empty.

4.1.12. <M>ail

--------------

Syntax: M <CR>

The MAIL-Command connects you to the nearest BBS as defined by the sysop.

This command therefore works like a "Connect" command with predefined

destination. The BBS callsign can be shown with "M ?" (notice the space!)

4.1.13. <MH>eard

----------------

Syntax: MH [options] <CR>

The MHeard-Command by default displays the last 30 direct heard callsigns.

Optionally, a port number, a callsign (with or without SSID) or a number

(16 ... 200) of entries to be listed, may be given.

"MH x" shows all entries that contain the "x" in the callsign i.e. "MH WJ"

shows the entries for DF4WJ and DK7WJ but also a WJ2O callsign.

4.1.14. <MY>call

----------------

Syntax: MY <CR>

The mycall command gives the callsign and the SSID range of the node.

Example:

=> MY <CR>

mycall: DB0ODW, SSID's: 0-7

=>

4.1.15. <P>arameters

--------------------

Syntax: P <CR>

The PARAMETERS command puts out a list of the current parameters and some

channel statistics. Additionally, the links as shown with the <L> command

are displayed.

Example:

=> P <CR>

po  id td  qso usr tifr rifr tkby  rkby  qty    mode     links

1   -- 10   30  1  365  287    50    33  100   9600d*+   DB0KT 0-10   6/6

2   -- 1    36  1  271  908    30   163   99  19200d*+   DB0GV 0-0    4

3   -- 1     1  1    0    0     0     0  100   9600d*+   DB0GV 6-6   10

4   -- 40    3  1   27    3     2     0   82   1200*+    DB0TCP 0-15 580/647

5   -- 1    50  1  835  377   102    55  100  19200dtr*+ DB0SHI 0-15  11/39

6   -- 1    39  1  582  546    78    42  100  38400d*+   DB0GV 10-12  1/1

7   -- 40    4  1   31    3     2     0   70   1200*+    DB0ASF 0-15 229/243

8    7 40    8  8  184   36    34     1   92   1200*+

The single columns mean:

po:      Port number

id:      Port SSID, on interlink-only ports "--"

td:      TxDelay in 10 ms units

qso:     number of QSOs on this port, internode QSOs are also counted

usr:     number of stations heard on this port (since 3 mins)

tifr:    transmitted I-frames within the last 10 mins

rifr:    received I-frames within the last 10 mins

tkby:    transmitted kilobytes within the last 10 mins

rkby:    received kilobytes within the last 10 mins

qty:     quality of the channel; this is updated every 10 mins, but not if

there was nothing to send.

mode:    Baudrate on this port, additionally:

"c"  KISS: CRC-Mode, HDLC: Software-DCD (depends on hardware)

"d"  fullduplex

"t"  external TX-Clock

"r"  external RX-Clock

"z"  NRZ mode

"m"  DAMA master

"s"  port is synchronized

"u"  port is user port

"y"  autosysop

"+"  8 Mhz CPU-Clock (RMNC)

"!"  12 Mhz CPU-Clock (RMNC)

"#"  16 Mhz   CPU-Clock (RMNC)

links:   see <L>-Command

When counting the I-frames, re-iterated frames and frames which got lost

due to DISC are not counted. The kilobyte statements are only the contents

of the acknowledged I-frames, re-iterations are not counted, too. Thus,

this is the genuine net data rate.

4.1.16. <Q>uit

--------------

Syntax:  Q <CR>

The QUIT-Command ends the QSO with the node. After a "73!" you get

disconnected. If you are connected from another FlexNet node, you will be

reconnected to that node.

4.1.17. <S>etsearch

-------------------

Syntax:  S <CR>

The SETSEARCH-Command displays all digipeaters via which the FIND-Command

searches for someone.

Example:

=>S<CR>

search digi's:

DB0ODW

DB0KT via DB0ODW

DB0AAI via DB0ODW

DB0DA via DB0ODW

DB0IE via DB0ODW

=>

The frame generated by the FIND-Command would be sent via

DB0ODW, DB0KT, DB0DA, DB0AAI and DB0IE.

4.1.18. <T>alk

--------------

Syntax: T <call> [<text>] <CR>

With this command you can talk to other users connected to the node. There

are two modes: If there is a text given behind the callsign, then this

line is sent and you get back to the prompt. Thus, you have to issue a new

Talk-Command for each line. By "T <call> <CR>" you get into the permanent

talk mode which can be left later by using "/q". This is similar to

convers mode, with the difference that it does not happen on a convers

channel. All Convers-Commands are active and the current status can be

displayed with "/c".

4.1.19. <U>sers

---------------

Syntax:  U [n] <CR>

The USERS-command displays all users which have a QSO with or via the

node. Additional information is provided:

Example:

=> U <CR>

1: S5    P0: DB0ODW>DG3FBL

6: S7 U1 P0: DB0ODW>DK7WJ

35: S5    P0: DL1AA>DB0GV v DB0ODW DB0KT

2014: S5    P8: DB0GV>DL1AA v DB0KT DB0ODW

i.e.

1. column:    QSO number. The node uses this number for internal

management of the QSO.

2. column:    QSO state. This number shows the state of the QSO.

(see appendix for explanation)

3. column:    shows the number of unacknowledged frames of the

QSO, if there are any.

4. column:    port

5. column:    calls and digipeater field

The QSOs with the node are shown first, then the ones which run via the

node. Additional parameters may be specified on the "U" command line. If

you enter an "i", only QSOs with the node are shown. If you enter a port

number, you get all QSOs via that port. Using "U *" you get additional

information about the QSOs. The parameters may be combined. For example,

"U * 4" shows all QSOs on port 4 with detailed information.

Example:

=> U * <CR>

1: S5        F100  M3   P0  : DB0ODW > DG3FBL

6: S7  U1    F87   M7   P0  : DB0ODW > DK7WJ

35: S5      ! F50   M4   P0  : DL1AA > DB0GV v DB0ODW DB0KT

2014: S5      ! F66   M7   P8  : DB0GV > DL1AA v DB0KT DB0ODW

Additionally, the actual FRACK time "Fxxx" and the MAXFRAME "Mx" are

shown for each QSO. On DAMA masters the DAMA priority is shown instead of

FRACK. A "!" in front of the F-value says, that the QSO is using header-

compression (see section 9.8).

4.2. Sysop-Commands

-------------------

In addition to the commands a "normal" user may use, the sysop has some

extra privileges. This includes additional commands, or additional

parameters to the user commands. Commands which have additional parameters

are the as followed:

<L>inks             setup of link partners

<M>ail              setup of nearest BBS

<MY>call            setup of node callsign and SSID range

<P>arms             setup of sundry parameters

<IO>                read inputs/set outputs

Additional commands are:

<CAL>ibrate         transmit calibration signal

<K>ill              kill a single QSO

<MO>de              setup HDLC parameters/reset L1-devices (SCCs)

<SY>sop             sysop authorization

<W>rite             write text-files (beacon- and search-files, too)

<TR>ace             monitor a port

<RESET>             cold reboot of the node

<RESTART>           warm reboot of the node

4.2.1. <CAL>librate

-------------------

Syntax: CAL <ch> <mins> <CR>

The CALIBRATE-Command turns on the TX of a specified port (parameter <ch>)

for one minute. While this time the carrier is modulated by a continuous

sequence of 0 and 1, producing a "diddle" tone with a 50:50 ratio. The

command is useful in 2 cases:

- It makes it possible for the link partner to set his antenna to the

right direction.

- The symmetry of the modulation may be checked and the modem maybe

adjusted for best results.

If there are frames in the buffer, they are sent before the calibration

starts. To trigger the PTT watchdog the CAL signal is interrupted every 15

seconds for a short time.

Optionally, the calibration time in minutes may be given.Default is one

minute. With the command "CAL <ch> 0" the CAL signal is cancelled.

4.2.2. <IO>

-----------

Syntax: IO <bit_no> 0|1 <CR>

With the IO-Command the output-lines of the reset-card may be set or

cleared. <bit_nr> specifies the number of the bit to be changed, and 0 or

1 says whether the bit should be set to "high" or "low".

4.2.3. <K>ill

-------------

Syntax:  K <QSO-No> <CR>

The KILL-Command terminates an existing QSO. The QSO number must be

specified (U-Command, 1. column). Why this command? It is not made to let

the sysop feel like the "big boss", but sometimes it is necessary to kill

a QSO.

Example:

Station A is connected to station B, and A transmits a longer text to B.

After a certain time, the receiver of the text, station B, gets busy, i.e.

his TNC sends RNR. When this state is not fixed by B itself, the QSO will

last for ever, at least up to the next blackout.

4.2.4. <L>inks

--------------

Syntax: L <ch|CALL|-> <CALL> [#|$|-|@|>|)|!] <CR>

The LINK-Command is used to set up interlinks. Two parameters are needed,

a 3rd one is optional. Callsigns may be written either as upper- or

lowercase.

1st parameter:

port:     set up direct interlink on this port

callsign: set up interlink via that callsign, i.e. the

destination is reachable via a neighbor already specified.

-       : the minus sign deletes the link entry

2nd parameter:

Here the callsign of the link partner is given. If no SSID's are

specified, 0-15 are assumed. When only the SSID 0 is to be linked, -0 has

to be added to the callsign.

3rd parameter: options

"#" link is not shown to the users, thus hidden links, for example for

service reasons, are possible .

"$" link is not checked for availability and not made known to the

network.

"@" no internode communication on this link , but may be used, if the

partner is not aware of the FlexNet protocol (i.e. mailboxes)

"-" partner is not made known to the network. This makes emergency- or

testlinks possible. Internode communication takes place, thus

destinations are routed, only the partner stays hidden.

">" Subnet-Link. This is used to set up subnets, which will receive all

information from the network, but are not made known to the network.

The partner callsign and the destinations from it are saved for

routing reasons, but not sent to the other network nodes.

")" Works like ">", but the link is hidden (">" + "#")

"!" no forwarding of Subnet destinations: Same like ">", but the

difference is that the "gateway" node is made known to the network.

It is possible to have more than one link to the partner on different

ports. The router will always use the best link available. You should

remember this if changes are made. The old entry may still be valid under

circumstances. It is also possible to link partners with the same

callsign, or a callsign covered by the SSID range of the node. This is

interesting for mailboxes, service computers, DX clusters and similar

systems. Using this feature, only the node is forwarded to the network and

not every single SSID of the different computers. This helps keeping the

D-list in the network smaller.

Example:

MYCALL DB0AIS 0-10

L 1 DB0AIS-8  @   (Mailbox)

L 2 DB0AIS-9  @   (Cluster)

L 3 DB0AIS-10 @   (TCP/IP)

Only DB0AIS 0-10 is known to the network. If there is a connect request to

DB0AIS-8, it is sent out on port 1 to the BBS.  The links are not tested

as usually. If the link is not available, the user gets connected to the

node itself. Here, the user should get to know what is wrong, perhaps by

the C-text. This routing method works on user ports, too. In our example,

if DB0AIS would have a user port, the node may be connected as DB0AIS or

as DB0AIS-3, and the BBS DB0AIS may be directly connected without

digipeaters.

More examples:

L 3 DB0KT

All frames to DB0KT will be sent on port 3

L 1 DB0KT

L 1 DB0FUL

On port 1, 2 link partners are reachable, so the setup has both calls on

port 1.

There is a principle which says that if no SSID's are specified behind the

link callsign, all SSID's are routed via this port. But when a SSID is

specified, only the SSID is routed.

Example:

L 1 DB0KT

All frames route to DB0KT, i.e. also the frames to DB0KT-1, DB0KT-2 are

routed to port 1.

L 1 DB0KT-7

Only the frames to DB0KT-7 are sent to port 1. Other SSID's are routed by

the D-List, when no other links to DB0KT are specified. For FlexNet

partners the SSID-Range is automatically adapted.

To remove an entry from the link list, a "-" is given instead of the port

number as the first parameter.

Example:

DB0ODW has the links

1: DB0KT

2: DB0EAD

3: DB0IE

"L - DB0KT" deletes the routing entry for DB0KT. If there is more than one

link to the partner, the command has to be given several times to delete

every entry. Always only the first entry is removed from the table. Links

to NET/ROM partners should be set up as shown in the appendix.

4.2.5. <MO>de

-------------

Syntax: MO <port> <mode> <CR>

This command sets the operating parameters of a specified port. The mode

parameters are:

<num>  baudrate (on internal clock)

"d"    fullduplex (halfduplex is default)

"t"    external TX Clock (depends on hardware)

"p"    (instead of "d") switches to fullduplex with a holding time of one

minute; the PTT watchdog must be disabled!

"r"    external RX Clock (depends on hardware)

"z"    NRZ mode (depends on hardware, NRZI mode is default)

"c"    KISS: CRC mode; HDLC: Soft-DCD (depends on  hardware)

"m"    DAMA master

"s"    synchronize channel with other "s"-channels

"u"    user port, activates for example TxDelay measurement

"y"    auto sysop: Stations which connect on this port without digipeaters

in their path are automatically sysops. For security reasons, on

RMNC this may only be defined in the EPROM.

"-"    deactivate port

"."    dummy, if there are no arguments needed on special hardware

The parameters baudrate, "d", "t", "r", "z" and "c" depend on hardware.

Check out the hardware or driver documentation.

Examples:

MODE 3 19200d      ;port 3 19200 baud fullduplex

MODE 3 38400trz    ;port 3 38400 baud ext. clock, NRZ

MODE 3 -           ;special case: turn off port 3

When a MODE command is entered, all Layer1 modules of all channels get

initialized. This is not problematic, only frames currently being

transmitted or received are involved. QSOs are not affected. A Mode-

Command may re-initialize "hanging" SCCs. Therefore you should always try

a Mode-Command first before a RESET or RESTART since all QSOs get lost in

this case. The port number is not relevant, a "MO 11" will  do the job.

4.2.6. <M>ail

-------------

Syntax: M <call> <CR>

With this command, a BBS callsign is assigned to the node, which can be

reached by the users issuing the "M"-Command. It must be reachable in a

single step and known to the autorouter.

4.2.7. <MY>call

---------------

Syntax: MY <call> [<ssid1> <ssid2>] <CR>

The MYCALL-Command is used to set up the callsign of the node. With the

optional SSID's a range may be defined by which the node can be connected.

The SSID range must include the SSID's of every port, no port SSID must

be outside the SSID range defined by MYCALL.

Example:

M DB0ODW 0 7

The node callsign is set to DB0ODW. The node may be connected as DB0ODW-0

to DB0ODW-7. When the MYCALL is changed, this will affect only new QSO's.

Existing connections stay valid under the old callsign. The internode

communication is re-initialized completely, since the change of the

callsign needs to be forwarded to the network immediately.

4.2.8. <P>arameters

-------------------

Syntax:  P I <value>

or  P S <value> <port>

or  P T <value> <port>

The PARAMETER command is used to set up the TxDelay, SSID and the node

timeout of a specified port.

"P I <n>"          sets the node timeout to <n> minutes where a range

from 60 to 255 is valid. n=0 (recommended) disables the

timeout.

"P T <n> <port>"   sets the TxDelay of port <port> to <n> in 10ms units.

"P S <n> <port>"   sets the SSID of port <port> to <n>. When an

already-set SSID has to be deleted, <port> has to be 16.

Why do we need SSID's ? They do two jobs: Only on ports which do have a

SSID, everyone is allowed to connect. Exclusive interlink ports therefore

should not have SSID's (exception: links to NET/ROM partners, see

appendix).

The SSID is also needed for routing purposes, if a user who is not in the

MHeard-list shall be connected on a specified port. The connect then needs

to go via the according SSID, i.e. via <nodecall>-<port-SSID>.

4.2.9. <RESET>

--------------

Syntax: RESET <CR>

This command cold-reboots the node. All QSOs, parameters in the RAM and

the text files get lost. The default parameters as stored in the RMNC

master EPROM are set. You should use this command only when your

parameters got disordered and you want to reset to EPROM defaults. This

command is available only on RMNC systems.

4.2.10. <RESTART>

-----------------

Syntax: RESTART <CR>

The RESTART-Command basically does the same as the RESET command. However,

parameters and text files remain in memory, so usually you should use this

command instead of RESET. You should use both commands only in emergency,

since all QSO's and the routing information get lost. This command is also

available only on RMNC systems.

4.2.11. <SY>sop

---------------

Syntax: SY <CR>

Starting with RMNC Version 3.3h and PC/FlexNet Version 3.3g the sysop

authorization has changed. The system now uses a procedure similar to

the BayCom BBS.

The SYSOP-Command is used to do the sysop authorization. When a remote

request is sent to the node from the sysop to enter the sysop mode,

the node answers with random numbers. These numbers correspond to the

characters in the password string of the parameter file (see 7.4.2)

or the password file in case of PC/FlexNet. The password string must

have a minimum length of 10 and a maximum length of 80 characters. It

may contain any printable character between $20-$7E and $A0-FE except

the quotation mark (").

How does it work ?

Assuming the password string would be the following:

"Thisisanicepasswordstringforournodeonthehill"

=>SY <CR>             <- enter sysop command

DB0DA> 41 12 34 7 16 <- numbers returned from the node

DB0DA answered with five numbers that correspond to the characters:

"hpdaw". You can either enter these characters followed by a <CR> or for

security reasons embed them in a larger string:

Gjdjg/hj7efjdencDfjefhpdawWfhjgflhlevBcne3d)fj <CR>

^^^^^

Now you gained sysop status (provided, your calculation was OK).

After a successful login as sysop no message is returned. You may now try

out whether it went all right using a harmless command like TIMEOUT. If you

are logged in as sysop, the node timeout is not valid for you anymore, i.e.

you may stay connected to the node as long as you wish. The sysop authoriza-

tion is removed by disconnect, reconnect (link reset) or a Connect-Command.

It is possible that more than one sysop is logged in at the same time.

4.2.12. <TR>ace

---------------

Syntax: TRACE <ch> [<call>] [<] [>] [#] [$] <CR>

By using the TRACE-Command, you can monitor the traffic on a given port.

This of course only works as long as the buffers do not overflow. When

your own QSO is fast enough, you may monitor for a longer time. Compressed

QSO's are shown only if your node is the source or the final destination

of the QSO. The command is cancelled if the buffers overflow or you type

another command. Only one sysop may monitor only one port at one time. You

should note that this command needs a large amount of memory and bus

capacity, which will slow down specially the monitored channel. Therefore,

you should not use it too often and under no circumstances permanently.

This command is only available on Solomaster systems.

Options:

#        do not display RR/RNR/REJ-Frames

$        suppress the I and UI texts, i.e. display only frame headers

<call>   trace only this callsign. Only source and destination callsigns

of a frame, i.e. no digipeater calls are checked. SSID is taken

into account if specified.

>        only frames sent

<        only frames received

4.2.13. <W>rite

---------------

Syntax:  WRITE <A|B|C|H|I|L|S> <CR>

By using the WRITE-Command, the texts for L(A)test news, (B)eacon, (C)-

Text, (H)elp, (I)nfo, (LO)cal and (S)etsearch may be entered. All texts

except (B)eacon and (S)etsearch may have any desired format. The C-text is

sent after the standard system prompt at the beginning of a connect.

Standard prompt is "xxxx/FlexNet Vx.x". The C-Text is shown after this.

After any Write-Command, the amount of available memory on the RMNC is

shown.

We recommend the following usage of the texts:

LATEST NEWS:  recent information about the node and things every user

should know

INFO:         general information about the digipeater. QTH, hardware,

antennas, use of IO-ports etc. You should not forget to

mention the user QRG's.

LOCAL:        this text is appended to the C-Text, when the user

comes direct, i.e. without digipeaters in the

path. This is the right place for information about

the channel access method and other information which is

only interesting for local users.

HELP:         A short user's guide to the system with the most important

commands

The text files cannot be saved in the EPROM due to space limitations. The

end of the text is marked by either /EX or Ctrl-Z. The text is saved up to

the last line before the /EX. Since many PR-stations use "split screen"

programs, it is recommended to begin every text except the C-Text with a

single <CR>. This looks much better.

The Beacon-file has a special format. You may set up any beacon on any

port. The file format is as follows:

# <t> <p> <tocall> [via [via ...] ] :Beacon

text..........................#

<#>       delimits the different beacon information

<t>       time between two beacon transmissions, in minutes.

(1..255 minutes)

<p>       port number where the beacon is to be sent

<tocall>  Destination call of the beacon, for example "BEACON", "RMNC",

"FLXNET", "TEST" or similar , there may be up to 8 digipeaters

specified, via which the beacon will be sent.

Example:

10 0 RMNC:Digi Odenwald * JN49IQ * Krehberg/Odw. *#

30 1 RMNC DB0KT DB0ODW:DB0KT QRV#

5  0

FLXNET:Testbeacon DB0ODW

Our example consists of 3 beacons, each delimited by a "#".

(beacon1...#beacon2...#beacon3...) Between the single statements there may

be <CR>s to improve readability. It is not important whether the callsigns

are upper- or lowercase. Source callsign of the beacon is always the

mycall of the node. When no beacon-text has been entered since the last

cold-reboot, the default-beacon is sent every 3 minutes:

#3 0 FLXNET:RMNC/FlexNet V3.3d

All beacons are sent as UI (Unproto-Information) with the command bit set.

The SETSEARCH-file has a special format, too.

There may be as many search paths as you like. The limit is dependent upon

the available memory. The number of the digipeaters is limited to 7.

The format is:

<call1>

<call1> [ <call2> [ <call 3> [ <call 4> [ <call 5> ]]]]

The SETSEARCH-file contains all paths via which the FIND-Command sends its

UI-Frames. The first callsign in the line is the digipeater which shall

send the UI to the user, the following digipeaters specify the path to the

destination digi. These paths should always be identical to the path a

user will use. This means, digipeaters on the route to the destination

should be left out, the autorouter will know the way.

Example:

DB0ODW

DB0DA via DB0ODW

DB0KT via DB0ODW

DB0AAI via DB0ODW

First line says that the UI-frame is sent via the local user port. Second

line demonstrates how a path to DB0DA is set up. DB0DA is reachable from

DB0ODW via autorouter.

==========================================================================

5.Command Overview

==========================================================================

Statements inside [] are optional.

5.1. User Commands

------------------

A                  - display text LATEST NEWS

B                  - display beacon-file

C                  - start conversation mode

/w               - list node users

/w n             - list convers users on channel n

/c               - display convers channel number

/c n             - switch to channel n

/s call text     - send private msg to user

/q               - quit conversation mode

C call [v] [digi]  - connect further on

D [*] [call]       - display destination table, path to destination

F <call>           - FIND-command, look for <call>

H                  - display text HELP

I                  - display text INFO

IO                 - In/Out - status

L                  - display Interlink information

LO                 - display text LOCAL

M [?]              - connect next BBS

MH [...]           - display Heard-list [selectively]

MY                 - display Mycall and SSID-range

P                  - display parameters and statistics

Q                  - quit, end of connection

S                  - SETSEARCH, display search-paths

T <call> <text>    - send text to other users

U [*] [port/"="]   - display user table [selectively]

5.2. Sysop Commands

-------------------

CAL <ch> [min]          - port <ch> sends calibration signal

IO <bit_nr> 0|1         - set Output bit nr. <bit_nr> to 0|1

K <QSO_no>              - kill QSO nr. <QSO_nr>

L <ch> <call>           - route <call> to <ch>

L <viacall> <call>      - route <call> via <viacall>

L - <call>              - delete link <call>

M <call>                - assign local BBS

MY <call> [ssid1 ssid2] - set mycall and SSID range

MO <ch> <x>             - set port <ch> to mode <x>

P I <x>                 - set node timeout to <x> minutes

P S <ssid> <ch>         - set SSID <ssid> on port <ch>

RESET                   - cold reboot

RESTART                 - warm reboot

SY                      - sysop authorization

TR <ch> [...]           - monitor port <ch> (solomaster only)

W <A/B/C/H/I/L/S>       - write text-files (end: /ex)

A                     - text LATEST NEWS

B                     - text BEACON

C                     - text C-Text

H                     - text HELP

I                     - text INFO

L                     - text LOCAL

S                     - text SETSEARCH

=========================================================================

6. Legal Terms   -   License Agreement

=========================================================================

The following restrictions have only the purpose to guarantee the quality

and the development of the software and, of course, to avoid commercial

use of it, except in special cases agreed on with the author of each

module. By experience, on uncontrolled distribution it happens that only

parts of the software or obsolete versions of it are spread. This results

into problems at the end-user's and into unnecessary and annoying check-

backs. Bad experiences cause the list to grow constantly...

- RMNC/FlexNet, PC/FlexNet and the accompanying utilities and the

documentation are products of Gunter Jost, DK7WJ. Exceptions are marked

as such. All rights reserved by the author. The user is granted the

right to use the software under the following conditions:

-   The software is used only for non-commercial purposes within amateur

(HAM-) radio. All other usages, especially in other radio services,

need a written consent by the author in each separate case.

-   The legal rules of amateur radio operation are strictly followed.

-   Commercial copying and sale of the software is not allowed without

prior written consent of the author.

-   No changes must be made at the software which are not explicitly

agreed with the author. This does NOT apply for setting specific

parameters.

-   Copyright marks and copyright messages of the software modules must

not be changed or removed.

-   The software must not be distributed via BBS systems or public

accessible bulletin boards, neither in part nor as a whole.

-   Neither the author nor the distributors of the software may be liable

for any damages, no matter of what kind, which may occur when using

the software. THE RISK OF USING THE SOFTWARE IS ENTIRELY YOUR OWN!

Under this conditions you may make as many copies of the distribution

disk as you wish and give them away, where you always have to state the

original source (FlexNet-Gruppe Darmstadt, G. Jost, DK7WJ).

The sourcecode of the software is not available.

6.1. Disclaimer

---------------

Neither the author nor the distributor of the software may be liable for

any damages that may eventually occur when using the software RMNC/FlexNet

or PC/FlexNet. The software is provided "AS IS", without warranty of any

kind, including but not limited to the warranties of merchantability and

fitness for a particular purpose. No features should be taken for granted.

The documentation may contain errors or mis-translations.

By using the software you agree to the conditions above.

==========================================================================

7. Installing  RMNC/FlexNet

==========================================================================

7.1. The RMNC

-------------

The RMNC consists of an port controller card for each port on a euro-sized

board (100 * 160 mm). Additionally a so called reset card is needed which

contains the system watchdog and the In/Out-ports for supervisor and

control reasons. The cards are run in master/slave mode, i.e. one card is

the master (meaning it doesn't carry a radio port), the other cards are

the slaves. The whole communication on the system bus is controlled by the

master, that means the master polls every slave for information and mediates

them where they belong. The only hardware difference between the cards is

that the master contains a different EPROM. This has the advantage that only

the master needs to be configured, all slaves receive their configuration

information from the master.

After a reset, the master determines the number and the addresses of the

attached cards. Then these cards get initialized with their parameters. If

there is a malfunction at one of the port controller cards, the master

gets to know this after the watchdog-reset which will occur. The defective

card will not be polled anymore, and the node will still work - provided

that the damage does not block the bus.

After the reset, all controller cards are ready to accept QSO's. A

connection via the digipeater counts as a QSO, too. The software on the

controller cards will manage the complete L2 connection on its own and

mediates all data directly to the according controller card.

7.2. The RMNC-Solomaster

------------------------

Since RMNC/FlexNet Version 3.3h a master is mandatory (see 7.1).

The expression "solomaster" is therefore obsolete.

7.3. Card Addresses

-------------------

When installing the RMNC/FlexNet software, the card addresses need to be

configured. Attention, the master card has no address, i.e. all DIP-

switches need to be open! For addressing the other cards, please refer to

the hardware manual. The cards may be addressed in any desired order, gaps

do not matter.

In the parameter table (P-command) the master always is port 0. When using

a Solomaster, then no port 0 is shown. Now the EPROMS can be installed on

their boards. Only ONE master EPROM has to be installed, all other cards

must be equipped with (identical) slave EPROMS.

The RMNC should be ready for use now. Immediately after the reset, the

first beacon is transmitted on the master port or on solomaster systems on

port 1.

7.4. Generating a MASTER EPROM

------------------------------

The software is distributed unconfigured. To configure it, you need at

least an EPROM programmer capable of programming 27C512 EPROMS, and an IBM

compatible PC.

7.4.1. Parameter Compiler

-------------------------

The master EPROM is configured with a PC program. With the aid of this

compiler and a parameter file, a configured binary file will be created

which is ready to be programmed into the EPROM. The slave EPROMS do not

need to be configured as they are the same on every RMNC. To get the

compiler to make the binary file, you have to create the parameter file

first. The parameter file is a plain ASCII file which can be made with any

editor. This file contains all necessary parameters. The syntax of the

file is similar to the syntax of the sysop commands. When you have

finished making the file, you can start the compiler. If you are lucky you

now have a file with the same name as that of the text file, but with the

extension ".BIN" in the directory. This file also exists when warnings

occur there. If the compiler detects syntax errors, no .BIN-file will be

created. For control reasons a second file with the extension ".LST" is

produced. You should have a look at it to ensure that the compiler

understood everything. The .BIN file can be burned into a 27C512 EPROM

now.

Syntax of the compiler:

MRMNC <parmfile> [<binfile>] <CR>

The first command line parameter is the file name of the parameter file

with extension. Specification of the <binfile>-name is optional and will

only be necessary if the name desired is different from the name of the

text file. If a file with the same name exists, the compiler will ask if

you want to overwrite it. The compiler creates a list file (".LST")

which contains all default parameters.

7.4.2. Structure of a parameter file

------------------------------------

Everywhere in the file, except inside of commands, comments are allowed.

Everything which stands behind a "*" or a ";" is ignored. Empty lines are

ignored, too. The syntax of the commands is the same like entering the

commands into the digipeater. The file is not case sensitive. It is

recommended that you note down everything carefully. This makes changes

easier later.

You should pay attention to the commands SPEED and END, which can only be

given in the parameter file, and not as commands online. The WRITE-command

does not work either, texts can be entered only online.

Differences to the normal syntax occur at the  SYSOP-command. Here the

password string for the node is entered.

Example:

* Configuration of DB0ODW

* date:  1st Apr. 1998

SPEED 8                    * The master runs at 8 Mhz clock speed

* (4/8/12/16 Mhz are possible here)

Mycall  DB0ODW 0 7         * Mycall of the node, SSID range 0-7

Sysop "Thisisthepasswordstring"     * Password string for sysop access

* must be be between 10 and 80 charak-

* ters; starting and ending with

* quotation marks

P SSID  0 1                * Userport is port 1 with SSID -0

P SSID  2 5                * 2nd Userport (-2) on port 5

L 2 DB0KT                  * Interlink to DB0KT

L 3 DB0AAC                 * Interlink to DB0AAC

L 4 DB0IE                  * Interlink to DB0IE

L 5 DK7WJ-8 #              * Testlink to DK7WJ-8 not to be forwarded

* to the network

IO 7 1                     * set IO-bit 7  (turn on PA to DB0IE)

mode 1 96t                 * Port 1 with 9600bd and external

TX-Clock

mode 2 96d                 * Port 2 with 9600bd fullduplex

mode 3 96d                 * Port 3 with 9600bd fullduplex

mode 4 24                  * Port 4 uses 2400bd

P I 0                      * No timeout for the node (Infobox)

* Layer1-parameters

parm TxDelay 25 1          * User ports need a higher TxDelay

p t 6 2                    * Interlinks only need 60 ms

p t 6 3

p t 6 4

parm TxDelay 25 1          * this is another Userport

END                        * The END command must be the last one!

7.5. Slave-EPROMS

-----------------

The slaves need not to be configured. The EPROM file RM_SLAVE.BIN is

intended for programming into an 27C128 EPROM. When a 27C256 EPROM is

being used, the file must be placed at the upper half, beginning at $4000.

7.5.1. EPROM Patch Slaves 9-15

------------------------------

The RMNC2 controller cards contain only a 3bit address switch, thus you

can only address a maximum of 8 cards (1 master and 7 slaves). For the

slaves 9 - 15, the byte $3F00 ($7F00 on 27C256) must be patched to a value

different to $FF. This adds "8" to the address setting of the DIP

switches. On RMNC3 cards this is not necessary anymore, here the address

can be set directly.

7.5.2. KISS-Slave

-----------------

It is possible to use the KISS protocol directly with the node. There is

no separate KISS EPROM anymore, on the RMNC2 you have to add a single wire

between pins 18 and 39 of the 6522. On RMNC3 you have to short the jumper

JP1. Since the KISS protocol is not secure, a CRC mode was added. It can

be activated by a MODE-command. Specifications of this new mode are

available from the author.

==========================================================================

8. Installing PC/FlexNet

==========================================================================

PC/FlexNet runs completely in the background as a TSR, that means that

other applications can run simultaneously if there is enough memory left.

However, the FlexNet infobox and the beacon generator may not be serviced

under some circumstances, so that a dialogue with the node is impossible.

This only happens when using badly programmed applications.

QSO's via the digi and the internode communication are not affected and

should always work, whatever the PC has to do. Probably things get slowed

down a little bit.

8.1. Hard- and Software Requirements

------------------------------------

-   PC/XT, better AT with at least 512kB RAM

-   PC/FlexNet needs 200kB RAM, plus space for the L1 drivers and

applications

-   Operating system MS-DOS 3.1, better 5.0 or 6.2. Tests with MS-DOS 6.0

caused problems, we have no experiences with DR-DOS or other DOS

versions. We recommend the use of MS-DOS 5.0 or 6.2, here most modules

can be loaded into the UMB's, provided there is enough memory

available.

-   IO-ports as necessary, according to the L1 drivers available

Principally, a PC/XT will work. The gained performance mostly depends on

the speed and the throughput of the L1 hardware drivers.

PC/FlexNet supports several loadable L1 drivers. They are installed in the

memory by simply calling them. This makes it easy to support any hardware.

A "driver development kit" for interested developers is available from the

author. The port numbers derive from the order of the driver installation.

A single driver can support more than one port depending on the hardware.

FlexNet, however, is limited to a maximum of 16 ports, the last port (15)

is reserved for internal purposes.

The port drivers are included on the distribution disk, depending on which

drivers are available. For every L1 driver there is a appropriate *.DOC-

file which explains the installation. By starting the drivers with the

option /?, you will get a short help as well.

Many people on different places are working on PC/FlexNet at the same

time. Thus, there always new versions of kernels, drivers and

applications. It is always a good idea to ask for new versions if there

occur any problems. Changes, even in the installation procedure, may

happen. Please read the *.DOC-files carefully!

==========================================================================

8.2. Installation and Configuration

==========================================================================

At the beginning, all files must be copied into a directory which should

be in the DOS search-path. The start of PC/FlexNet should be done via a

batch file because most of the L1 drivers need additional command line

arguments. Occurring errors should abort the batch file. A sample batch

file is on the distribution disk and can be easily changed to fit your

needs.

FLEXNET.EXE must be loaded first, then - if a node is to be installed -

FLEXDIGI.EXE. Pure endpoints (Terminal, Cluster, BBS and so on) should not

use it. Then the L1 drivers follow in the order you require. At last, the

activation of the modules is made by the utility "FLEX". After doing this,

no more port drivers can be installed.

FLEXNET.EXE has an optional parameter, which specifies how many RAM may be

used by FlexNet. Default is 15kb, but this lasts only for few QSO's. The

minimum for nodes with several ports is about 80kb. Depending on how many

ports you use, you should experiment with this value. FlexNet loves memory

more than everything else and runs best when it has about 30kb per port

and additional 20kb for administration.

To load the modules, generally (from DOS 5.0 onwards) you should use the

"LOADHIGH" or "LH" command. It does not do any harm if there is not enough

memory in the UMB; the file is loaded into conventional memory then. You

still gain a little memory, since the environment blocks do not fragment

the memory. You may check this by using the "MEM /D" command.

Calling FLEX.EXE with the argument "/U" uninstalls all L1 drivers and

removes FLEXNET.EXE from memory. As usually on DOS, no other TSRs should

be loaded after FlexNet, otherwise your machine might crash.

The first start of FLEXNET.EXE creates an empty parameter file.

Port 15 is generally the interface for applications. The parameter

AUTOSYSOP ("y") is set on this port, you should not change it. Now you

should set the sysop secret code using "FLEXPASS.EXE". With "TNC.EXE" you

can connect the node and continue in setting the parameters. If you made a

mistake, you could simply delete the file "FLEXNET.FPR" and begin again.

"TNC.EXE" is a simple TNC emulation. With "<ESC> H <CR>" you get a short

help. The node can be connected with "<ESC> C <CR>".

The parameter setting of the software can be done now either by the

TNC emulation or via remote control. Please check the documentation of the

L1 port drivers. Like always on FlexNet, the rest of the parameter

settings is very easy and can be finished in a short time.

A remark to the utility "FLEXPASS.EXE": this utility needs an ASCII-

file as an argument that contains the password string.

Before you decide to build a digipeater using PC/FlexNet, you should

think about the following: The RMNC is still the preferred platform for

FlexNet, and something that does not work there will not work on the PC

either, except from some bagatelles. The user shall find a uniform and

well known (from the RMNCs) user interface. Who prefers the optimum of

reliability and performance for minimal costs and maintenance should use

the RMNC.

==========================================================================

9. Appendix

==========================================================================

9.1. Protocol version AX25 V2

-----------------------------

FlexNet understands QSO's with AX25 version 1 and AX25 version 2. A QSO

with the node itself can only be done using version 2. When the node

encounters a SABM to it with V1, it replies with DM. When station A is

trying to connect a station B via the digi using V2, and the other station

responds with V1, the connection happens in V1. However, hop-to-hop

acknowledgement is disabled for this QSO, it is not even mentioned in the

user list.

9.2. Unproto-Frames

-------------------

All UI- (unproto) frames are transmitted and routed, even frames using

protocol version 1. This makes it possible to run TCP/IP without any

problems, since most TCP/IP programs send their UI-frames in AX25 V1. The

length of the I-field of the UI-frames must not exceed 256 bytes.

9.3. RNR-Behavior/Recovery

---------------------------

Because the memory of the node is limited, it may happen that a user is

set to RemoteBusy (RNR) from the digipeater. FlexNet works very

defensively in this matter to ensure that there is not too much memory

used for one single QSO. It makes no sense anyway to buffer more than

about 10 frames for one QSO. RNRs therefore do not mean that the memory of

the node is short, but only that there are enough frames buffered to

service the QSO fluently. This ensures a fair partition of the channel

resources to the QSOs. RNRs often happen in convers mode when one of the

partners has a slow link. When the RNR condition has gone, the braked

station gets a RR frame with the poll bit set. This causes a RR-final from

the station, but only this ensures that the status change is detected by

that station. On many AX25-implementations only a RR without poll is sent,

which causes hangs of several minutes when it gets lost.

9.4. Reconnects

---------------

A special problem on digipeaters with hop-to-hop acknowledgement are

reconnects. A reconnect does mean that a station tries to (re-) establish

another connect to its partner using the same calls (and SSID's) and the

same or a different path. When this connection happens on a different

path, problems may arise. The digi holds the running QSO in his table.

When there are no unacknowledged frames, nothing happens. The "zombie-QSO"

dies after 15 minutes. However, if there is still data to be sent, the

digi tries to service the other station. But since the QSO was re-

established using the other path, the sequence numbers of the original QSO

do not match with the new ones. This causes a FRMR (frame reject) and the

connection is aborted. The best way to avoid this trouble is to end an

existing connection properly before establishing a new one.

9.5. I-Polling Method

---------------------

During a connection, it often happens that an I-frame is not acknowledged

correctly. In this case, AX25 V2 says that a poll frame (RR) has to be

sent in order to query whether the I-frame was received correctly. If not,

the I-frame is retransmitted, otherwise the next frame is sent. However,

it is provable that this method causes a useless load to the channel, when

the I-frame in question is a very short one. With the I-polling method,

the short I-frame is repeated immediately with the poll bit set. This is a

mixture of the old V1 and the new V2-protocol version. For statistical

reasons, the threshold of this method is set to 40 bytes. Any I-frame with

fewer than 40 bytes length is repeated immediately when it was not

acknowledged within the FRACK-time (T1). The I-poll method is only used at

the first 3 retries, then normal polls are applied.

9.6. Channel Access Method

--------------------------

Up to FlexNet V3.1 the p-persistence algorithm was used. This had the

disadvantage that the send-willingness of the station could only be

adjusted manually. Since V3.2 there has existed a fully adaptive channel

access method, which adapts itself by determining the number of users and

the channel allocation to calculate an optimal compromise between

throughput and the collision-probability.

Variables used:

B   Baudrate (1/s)

TxD Tx-Delay (s)

R   Random number (1 .. 10)

U   Total of the users on the channel

K   channel allocation

t1  waiting time after transmission (s)

t2  waiting time between two TX-tries

19200/s

K = (-----------  + 1 ) * (U + 1)

B

If K > 255, then K is set to 255. This prevents an increase of t1 on more

than twenty users on 1200Bd.

t1 = K * (R+1)*8 * TxD

If t1 > 10s, then t1 is set to 10s, this can only happen on baudrates

below 1200Bd.

t1 starts when its own transmission has ended. t1 is stopped as long as

the channel is busy. As long as t1 is active, the TX is blocked. When t1

has expired, and the node wants to transmit, the DCD is checked in t2

intervals. If there is only one user, t2 becomes 0.

t2 = 2*TxD * (U-1)

When the channel is busy after the expiration of t2, t2 starts again,

otherwise the transmission is started. t2 is not interrupted by the DCD.

9.7. Layer 2 States

-------------------

The second column of the user list shows the actual L2 states. They shall

be briefly explained here. For more information, please refer to the

AX25-V2 protocol specification. Attention, we are using other numbers for

the single states. Only the numbers 1-7 are the same as in the

specification. The other states, which are only relevant for BUSY (NR)

states, have been done by using bit masks, i.e. there are offsets added.

State        Description

------------------------

1            disconnected

2            link setup

3            frame reject

4            disconnect request

5            information transfer

6            REJ frame sent

7            waiting acknowledgement

Disconnected

------------

This state is the default state. No QSO exists at the moment, therefore it

is not shown in the user list.

Link Setup

----------

A connection is being set up, i.e. SABM-frames are being transmitted, the

acknowledge (UA) has not been received yet.

Frame Reject

------------

Due to a sync error a FRMR-frame was sent, the connection is restarted.

This may happen due to a software error or due to two stations with the

same callsign.

Disconnect-Request

------------------

A connection is being disconnected, i.e. DISC-frames are being sent. The

acknowledge (UA) has not been received yet.

Information Transfer

--------------------

Hopefully, the most seen state. The connection is established, info-frames

are being exchanged.

REJ Frame sent

--------------

A REJ-frame was sent, because a received frame was not in the correct

order. A retransmission is requested.

Waiting Acknowledge

-------------------

The link is being polled, either because I-Frames have not been

acknowledged or because the link has to be checked.

Busy-States

-----------

When the station is busy, i.e. cannot receive any more packets, 8 is added

to the state number. When the opposite station is busy, 16 is added to the

state. When both stations are busy, 24 is added. Thus, numbers up to 31

may occur.

9.8. Header Compression

-----------------------         --> Only available on Solomaster systems!

In the address field of the L2 protocol, up to eight digipeaters can be

specified, which identify the path of a frame. In contrast to other node

concepts (Net/ROM), FlexNet uses these fields for the routing of the frame

to avoid a separate L3 header. All routing information is put into this

address field. A disadvantage is, that this address field may grow very

long, since every callsign allocates 7 bytes. Due to the VirtualCircuit-

concept, however, this becomes redundant when a L2 connection is

established. So with FlexNet V3.2 a compression of these headers was

introduced. Instead of the 14 - 70 Byte, the header is now only 7bytes

long, which causes a significant increase in performance. Especially short

frames (RR, short I-Frames) shrink to 10 - 25% of the original length,

which speeds up some interlinks to the double speed! Of course, this

method can only be used if both partners of the interlink are capable of

the protocol used. Address fields are only transmitted completely during

link setup, during the QSO itself only the shortened addresses are used.

Another shrinking of the header would be possible but could be dangerous

if used on non exclusive channels. An obvious callsign needs to be

transmitted to avoid mistakings. During the link setup, the own QSO number

is sent behind the control field as a 14bit integer. As opposite to the

address field, this number is transmitted right-justified. In the UA

frame, the QSO number is transmitted also. Now every partner knows the QSO

number of the other node. As soon as the connection is established, the

compression kicks in. In the compressed address, the callsign and the QSO

number of the receiving digipeater is transmitted.

Format of the compressed addressfield:

--------+---+---+---+---+---+---+---+--------

Flag    ! H ! L ! C ! C ! C ! C ! C ! CTRL   ........

--------+---+---+---+---+---+---+---+--------

^-- normal control field

^---^---^---^---^------- compressed callsign + SSID

^---^--------------------------- QSO number + C-bit + final-bit

format of the callsign (example DB0ODW)

byte0000000011111111222222223333333344444444

bit 7654321076543210765432107654321076543210

ddddddbbbbbb000000ooooooddddddwwwwwwSSID

Every character represents the ASCII code minus 0x20. In the first both

bytes of the address field, the QSO number, C-bit and F-bit are encoded.

The explicit setting of the Final-bit (bit 0) makes sure that the frame is

not confused with the conventional AX25 address field.

7 6 5 4 3 2 1 0 7 6 5 4 3 2 1 0

q q q q q q q q q q q q q q C F   -   final-bit (always 1)

^------   command/response

command = 1

response = 0

^-^-^-^-^-^-^-^-^-^-^-^-^-^--------   QSO-number (shifted left 2 bit)

Specialities:

-   Internode communication generally runs uncompressed. This would be

very difficult to implement, since the ability of doing compression is

transmitted within the internode communication. Compression here is

not too useful, since there are no digipeaters in the path, and if,

compression could not be done. Besides, this ensures the transmission

of the callsigns in regular intervals to comply the laws.

-   When for a running QSO a single conventional (uncompressed) address

field is received, this QSO has to be switched to uncompressed mode

(exception: link reset SABM with compress offer).

-   When an internode reset is received (O-Frame), then ALL QSO's to this

node are switched to conventional addressing mode.

-   At the beginning of the internode-QSO it is announced in the O-frame,

whether compressed addressing is possible. It is only used when both

nodes are capable of it.

-   Both free bits in the SABM and the UA are always set to 0 to allow

later extensions.

-   When receiving a number per SABM, it has to be checked if this number

ia  already used in an other QSO. This may happen when the originating

node had a reset. In this case, the older QSO has to be killed

immediately.

-   When a compressed frame is received for uncompressed QSO, this frame

is discarded. This may happen if there was a reset and the QSO number

is now given to an other QSO.

-   Compressed SABM's or UI's are not generated but ignored: SABM's should

always contain the complete path. UI's can only very seldom be treated

as a QSO, so compressing the callsigns is of no use.

-   Header compression is only available on Solomaster systems. If you do

not like it, you may deactivate it by using fullmaster software.

9.9 Clock options

-----------------

9.9.1. Operating modes of the SCC

---------------------------------

The ZILOG 8530 SCC contains two separate serial ports, every channel has

its own clock generator. Since a HDLC channel needs two different clocks

(x and 32times x), the clock generator of the second port (port B) is used

to generate the single clock(x). Thus, port B is not usable for data

transmission. If you had used both ports for data transmissions, the RX-

Clock would have needed to be generated externally and additional parts

would have needed on the card. When using RMNC2 cards with their own AFSK

modem, the clock is derived from the PCLK supplied from the reset card

(RMNC3 has a local oscillator). Port A of the SCC is used for data

transmission. The clock generator A supplies the clock for the DPLL of the

receiver(32*x), the clock generator B supplies the TX-Clock(x). On RMNC3,

the A-port supplies the 32*x-Clock needed for the FSK modem. The TX-Clock

is generated by the modem. Thus, "external clock" needs to be set here.

9.9.1.Clock Options

-------------------

During the development of RMNC3 it proved necessary to change the clock

options. Especially affected is the combination of external TX and

internal RX clock. However, who uses some of the clocks on the RMNC for

attached modems should have a close look at the following table. The new

options and pin functions are as follows:

Mode:  --: internal RX and TX       I: Input

t: external TX              O: Output

r: external RX

tr: external RX and TX       SCC-Pins: TRxCA=14, RTxCA=12, TRxCB=26

Mode | TRxCA | RTxCA | TRxCB  |   comment

=====+=======+=======+========+=======================================

--  | O F32 | I TxC | O TxC  |  short-circuit TRxCB - RTxCA needed!

-----+-------+-------+--------+-----------d---------------------------

t  | O F32 | I TxC | O F32  |  ATTENTION NEW: Put clock to RTxCA !

-----+-------+-------+--------+---------------------------------------

r  | O TxC | I RxC | O F32  |  as usual, but swapped the clocks

-----+-------+-------+--------+---------------------------------------

tr  | I TxC | I RxC | O F32  |  no changes

----------------------------------------------------------------------

On RMNC2, almost everything stays as usual. On RMNC3, internal clock is

set, when the AFSK modem is used. When using the FSK modem or the AFSK

option with echo, external clock must be used.

When using an external modem, either Tx and/or Rx-Clocks can be supplied

from outside. Between these options you can switch by using the MODE-

command. Since the external modem must supply the single Rx-Clock, the

internal DPLL of the SCC is not used. The different clocks are injected

via the modem-disconnect connector. Take care, between RMNC2 and RMNC3

there are different mappings ! On RMNC2 TRXcA is the input for the

Tx-Clock (pin 16 of the modem connector), RTXcA is the input for Rx-Clock

(pin 18). On the RMNC3, there are only TxC and RxC. Be careful about the

following, too: Under normal usage (internal modem), TRXcA/TxC is an

output line, when using an external modem, this line becomes an input

line! If you have connected an external modem, and you switch to normal

mode, probably two outputs work against each other.

On the RMNC2, RTXcA is short circuit with TRXcB. Via this circuit the A

DPLL gets its 32x-clock. When using an external modem, you have to cut

this connection.

9.10. Links to Net/ROM partners

-------------------------------

FlexNet V3 makes it possible to link Netrom neighbors and to forward these

interlinks, provided they are working. For the following explanations, we

assume that our FlexNet node DB0FLX has an interlink to the NETROM node

DB0NR. The Netrom node consists of several TNC's with different SSID's.

The user port usually has the SSID -0. DB0FLX is on DB0NR-4 for example.

Obviously, we should add DB0NR-4 to the link table of DB0FLX as it was

done in V2. However, this has disadvantages:

-   The Netrom node perhaps has other FlexNet neighbors. If they forward

their DB0NR-x information, too, DB0NR appears more than once in the

FlexNet D-tables. This makes the D-tables very uncomprehensive.

-   When a user wishes to connect DB0NR, he does not know which path to

DB0NR the best is, since FlexNet may route the different DB0NR-x a

different way. However, it is possible to solve the problem with

proper entries in the link list. The idea is, that for the network it

is enough to know that DB0NR exists. How the single TNC's of DB0NR are

reached is only of interest for the neighbors of DB0NR. The link entry

of DB0FLX has to be as follows (provided that DB0NR is on port 5)

L 5 DB0NR-4 -     *  the "-" means that the link is checked,

*  but not forwarded to the network

L DB0NR-4 DB0NR   * this is the trick: The link to the user

* port DB0NR-0 is checked and forwarded to

* the network as ssid-range 0-15

FlexNet now knows a way to DB0NR, but only DB0FLX knows that it has to go

to DB0NR-4 directly, and to the other SSID's of DB0NR via DB0NR. In the

D-lists only a DB0NR (0-15) appears. The user port DB0NR-0 is directly

reachable. If there would exist a Convers-TNC DB0NR-8, it could be

directly reached, too. The thing becomes even more Wizzardly when DB0NR

has a second FlexNet neighbor. We assume DB0ZZZ is reachable via DB0NR-2.

We add the following two link entries:

L DB0NR-4 DB0NR-2 $           * indirect link. Not tested, not forwarded,

but displayed in the link list

L DB0NR-2 DB0ZZZ              * Link to FlexNet via DB0NR. Will be tested

and used by FlexNet.

That has been the good news, and now the bad news:

-    The D-command shows a route to DB0NR-14 now, even if it does not

exist. The Link-command is only capable of linking single (-0 -1 -2)

or all (0-15) SSID's...

-    The Netrom linkports have to turn on L2 digipeating. When accessing

DB0NR-0, this should cause no trouble, since the diode matrix works

without collisions.

Furthermore, the link as set up via this trick suffers from the not

available hop-to-hop-acknowledgement. This compromise is acceptable when

looking at the advantages of the network: When the links are working 100%,

it has no disadvantages, hop-to-hop-acknowledgement is not necessary if

there are no RETRIES. If the link is bad, FlexNet uses a better route, if

available. When the Netrom sysop is stupid and still wants L2 digipeat

turned off on the interlinks, the link still should be set up as

described. Then the NETROM node is not forwarded and the users have to

find their way themselves. As soon as the digipeating is turned on,

everything will work OK again.

9.11. Subscription Service

--------------------------

For the FlexNet software, a subscription service is available to sysops.

All sysops get always the new versions without looking out for it

everywhere. This service is provided free of charge, but cannot be

guaranteed. It depends on whether there come in enough donations for

funding this service, also there is not always enough time to do it. If

you are interested in subscribing, ask the author.

9.12. Correspondence

--------------------

The FlexNet-Group of Darmstadt, and therefore the authors and the

maintainers of the software are reachable by mail :

FlexNet-Gruppe Darmstadt

Gunter Jost, DK7WJ

Lichtenbergstr. 77

D-64289 Darmstadt

Germany

Or via AX25 Email:   DK7WJ@DB0GV.HES.DEU

The manual was translated by Mario Lorenz, DG0JAB @ DB0JES.#SAA.DEU.EU,

Internet-Email: ml@donald.bsz.szb.sn.schule.de. As I am not a native

English, this translation surely contains mistakes. Please send bug

reports or suggestions to the my Email-address. Thanks.

==========================================================================

10. Index

==========================================================================